Remaining Linear feet of welding.

Shell to Floor:

Half 1 inside 3/8” fillet weld (2) passes 53 ½’ each total = 107’

Half 1 outside ¼” fillet weld (2) passes 53 ½’ each total = 107’

Half 2 inside 3/8” fillet weld (2) passes 53 ½’ each total = 107’

Half 2 outside ¼” fillet weld (2) passes 53 ½’ each total = 107’

Top to Shell:

Half 1 outside 1/8” fillet weld (1) pass 53 ½’ each total = 53 ½’

Half 1 inside 3/16” fillet weld (1) pass 53 ½’ each total = 53 ½’

Top:

Side A Half is the drop down side 3 pieces and each is 39’ with 3/8” fillet welds inside and out.

2 passes each side 4 x 39’ total = 156’

Side B Half has just long seam to finish welding at 34’ needing 2 passes to finish.

2 passes x 34’ total = 68’

Mandoor:

Inside full pen with 3/8” fillet (3) passes x 16’ total = 48’

Outside full pen with 3/8” fillet (3) passes x 16’ total = 48’

Agitator Sulzer Rings:

Ring 1

Inside full pen with 3/8” fillet (3) passes x 12 ½’ total = 37 ½’

Outside full pen with 3/8” fillet (3) passes x 12 ½’ total = 37 ½’

Ring 2

Inside full pen with 3/8” fillet (3) passes x 12 ½’ total = 37 ½’

Outside full pen with 3/8” fillet (3) passes x 12 ½’ total = 37 ½’

Ring 3

Inside full pen with 3/8” fillet (3) passes x 12 ½’ total = 37 ½’

Outside full pen with 3/8” fillet (3) passes x 12 ½’ total = 37 ½’

Stiffeners:

Tall shell stiffeners

(6) at 12’ tall with 3/16” fillet per side (single pass) 6 x 12’ x 2 total = 144’

Short shell stiffeners

(6) at 9’ tall with 3/16” fillet per side (single pass) 6 x 9’ x 2 total = 108’

Drop down roof stiffeners

(8) at 4’ tall with 3/16” fillet per side (single pass) 8 x 4’ x 2 total = 64’

Nozzles and miscellaneous items:  additional 20’

Total remaining linear feet of welding 1416’