  - RECEIVING INSPECTION

    - All Code material received at the shop or field site is placed into a hold area pending receiving inspection.

    - The Quality Control Inspector or Receiving Clerk, using a copy of the Purchase Order for the P-Drive examines all Code material for condition, quantity, and identification marking to provide acceptability to the Purchase Order.

    - A Receiving Inspection Report (Exhibit 4) is completed for all Code material.  The Quality Control Inspector or Receiving Clerk will record the dimensional findings and all existing material identification markings on the Receiving Inspection Report.  Upon completion he will initial and date and forward it to the Quality Control Manager.

    - Weld material may be received as stock, and may be received without the use of a Receiving Inspection Report.  If this method is chosen, the Quality Control Inspector will verify proper material marking on the container against the applicable specification requirements listed in ASME Code Section II Part C.  If acceptable, he will place the weld material into the code weld storage.  The act of placing the weld material into storage releases it for production.

    - For product forms which the code of construction requires material test reports, the Quality Control Manager reviews the material test report against the requirements listed in ASME Code Section II.  If the material test report is acceptable, he will approve the material test report.

    - The Quality Control Manager shall verify the original product marking against the material specification or material standard as applicable.

    - If the product marking, and if applicable, the material test report meets Code requirements the Quality Control Manager will approve the Receiving Inspection Report.

    - Upon approval of the Receiving Inspection Report, the Quality Control Inspector will mark the material with the Purchase Order number and heat number and either place the material in stock, or if ordered for a job he will also mark the material with the job number and release the material for production.

    - Small items that are marked with the required original material identification may be placed in bags, boxes, or bins until assigned to a job in lieu of marking them with a Purchase Order number.  Prior to releasing to a job, the Quality Control Inspector shall mark these items with the job number.  If the item is too small, the job number shall be identified on the container.

    - The Quality Control Manager shall ensure that tests are performed and documented if further material testing is required to be performed at receiving inspection or during the manufacturing operations.

    - Material that does not meet all Purchase Order and Code requirements is retained in the hold area and controlled as described in Section 8 of this Quality Control Manual.