NONCONFORMANCE REPORT

| NCR No. | 002-26 |  |  |  |  |  |  | DATE: | 1/7/2026 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JOB No. | 089-25 |  |  |  |  |  |  | OTHER: |  |  |  |  |
| ITEM DESCRIPTION: |  |  |  |  | Stitch Cut Hole For Nozzle 1a Incorrect Location |  |  |  |  |  |  |  |
| DESCRIPTION OF NONCONFORMANCE: |  |  |  |  |  |  |  |  |  |  |  |  |
| There was miscommunication between Savannah Tank and the customer on the correct location of one of the nozzles. The incorrect nozzle designation was put on hold waiting for clarification on proper location. Actual nozzle being verified was stitch cut at incorrect location. Nozzle N1a (Pump Suction Connection). Moving from 47° to 45°. Savannah Tank Drawing 089-25-1. |  |  |  |  |  |  |  |  |  |  |  |  |
| QC INSPECTOR: |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDED DISPOSITION |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ USE-AS-IS |  |  |  | ☒ REPAIR |  |  | ☐ RETURN TO VENDOR |  |  | ☐ OTHER |  |  |
| Stitch locations will be welded from one side. Background and dye penetrant examination performed and filled from opposite side to result in a full penetration weld. Final weld will be Dye Penetrant inspected as well as RT examined after completion. Correct nozzle location will be marked on base material and hand cut. |  |  |  |  |  |  |  |  |  |  |  |  |
| QC INSPECTOR: |  |  |  |  |  |  |  |  |  |  | DATE: |  |
| DISPOSITION APPROVAL/ACCEPTANCE |  |  |  |  |  |  |  |  |  |  |  |  |
| ENGINEERING APPROVAL:(IF REQUIRED) |  |  |  |  |  |  |  |  |  |  | DATE: |  |
| QUALITY CONTROL MGR APPROVAL: |  |  |  |  |  |  |  |  |  |  | DATE: |  |
| INSPECTOR ACCEPTANCE: |  |  |  |  |  | N/A |  |  |  |  | DATE: | N/A |
| DISPOSITION COMPLETED |  |  |  |  |  |  |  |  |  |  |  |  |
| COMPLETED DISPOSITION AS RECOMMENDED? Yes ☒ No ☐ COMMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |
| QUALITY CONTROL MGR APPROVAL: |  |  |  |  |  |  |  |  |  |  | DATE: |  |
| INSPECTOR ACCEPTANCE: |  |  |  |  |  | N/A |  |  |  |  | DATE: | N/A |